Suraksha Chemicals Pvt. Ltd.

Unit 4 — Specialty Chemicals & Intermediates, Ambarnath, Maharashtra

PPE POLICY — PLANT FLOOR

Document SOP-PPE-01

| Document No. | SOP-PPE-01 |
| --- | --- |
| Issued By | Meera Iyer, Safety Officer (EHS) |
| Effective Date | 01-Jan-2024 |
| Applies To | All personnel and contractors, Unit 4 plant floor |

# 1. Purpose

This policy defines the minimum Personal Protective Equipment (PPE) required in each zone of Unit 4, and the additional PPE required for specific high-hazard tasks. PPE is provided by the Company at no cost to employees.

# 2. Baseline Requirement

Safety glasses, steel-toe safety footwear, and a hard hat are mandatory in all Process Bays and the Utility Block at all times, regardless of task.

# 3. PPE Requirement by Zone

| Zone | Mandatory PPE |
| --- | --- |
| Process Bay 1 (Reactor R-220) | Safety glasses, hard hat, steel-toe boots, chemical-resistant gloves, face shield |
| Process Bay 2 (Pump P-204A) | Safety glasses, hard hat, steel-toe boots |
| Process Bay 3 (Heat Exchanger HX-302) | Safety glasses, hard hat, steel-toe boots, heat-resistant gloves |
| Utility Block (Compressor C-101, Boiler B-501) | Safety glasses, hard hat, steel-toe boots, hearing protection |
| Storage Tank Farm (Valve V-118) | Safety glasses, hard hat, static-dissipative footwear, chemical-resistant gloves |
| Administrative Block | No mandatory PPE beyond visitor badge |

# 4. Task-Specific Additional PPE

| Task | Additional PPE Required |
| --- | --- |
| Hot work (welding, cutting, grinding) | Welding helmet/goggles, fire-resistant coveralls, leather gloves |
| Confined space entry | Full-body harness, supplied-air respirator (if indicated) |
| Working at height >1.8m | Full-body harness with double lanyard |
| Electrical work (LOTO) | Insulated gloves rated to system voltage |

# 5. Non-Compliance

Any individual found not wearing mandated PPE for their zone or task will be required to leave the area immediately. Repeated non-compliance is addressed under the Unit 4 disciplinary procedure (Plant Safety Policy Manual, SC-SP-001, Section 25).